# A sample article title

Text for this section.

Text for this section.

# Introduction

There are in general two types of mobile networks simulations in use. The first one is used during the design and development stage of the mobile network. These simulations are used to test individual parts, sub systems or the whole network. They are usually done when a new standard for a mobile network is defined or when new parts are developed. On the other hand, there are simulations that focus on the operation of the network. These simulations are used to investigate how the subscriber behavior affects the mobile network. The investigations can either be done in a virtual network to develop and test new algorithms and approaches or in a real network to test it under certain scenarios. Both, the virtual and the real network simulation require moving subscribers in the research area.

Our research focuses on real network simulations and how to estimate the subscribers mobility. These simulations use a model of the real network together with the subscriber mobility. Therefore it needs information about the environment (cell site location, soil conditions, and elevation profiles) and the subscriber mobility. The mobility can either be derived from a stochastic process or from real subscriber behavior. In our work we are using events captured in a core network to estimate the mobility of real subscribers in the network. We analyze handover events in order to estimate the subscribers route and its velocity.

The Austrian network operator A1, provided us with data of active (connected-mode) GSM speech subscribers. This is due to the fact that only active subscriber issue handover events. However, the framework can also work with idle subscriber and data (2G, 3G and 4G) subscribers for which location area updates exist instead of handover updates. In our work we focus on handover events, since these events have a better location accuracy.

Another part of our work is to model the real network as good as possible with freely available datasets, since mobile network operator are not wiling to share these datasets with third parties. These datasets contains the BTSs configuration, antenna characteristics, antenna tilt, etc. We are investigating how well a mobile network can be modeled without having to rely on the network operator itself.

We found out that, despite the fact that we are using freely available datasets, trajectories can be estimated for urban as well as semi-rural areas. Our approach can also estimate the velocity of the subscriber on the road network and is therefore well-suited to describe its mobility.

# Related Work

Over the past years research has been conducted in the field of mobility simulations and investigation of mobility related events in mobile communication networks. Besides mobility simulations and mobility modeling, mobile subscription data is also used for travel time estimation in ITS applications. We summarize fundamental concepts in both mobility simulations and travel time estimations.

#### Mobility Simulations

To simulate the mobility in mobile communication networks, stochastic processes are used. The mobility behavior of subscribers is modeled as a stochastic process. The most common mobility models are random way point (RWP) or random walk models such as Brownian motion (BM)  , Levy walks and flights  as well as the Markov mobility. Rhee et al.  compared the Levy walk model, which is an extended random walk, with the human mobility. GPS traces of 101 volunteers were used to compute the similarity. Their findings indicated that outdoor Levy walks with less than 10 km contain statically similar features to the human walk. To understand human mobility patterns Marta C. Gonzalez et al.  analyzed 100000 anonymized trajectories of mobile subscriber. They found out that human mobility is characterized by a time-independent travel distance and that they travel to a few highly frequented locations. However, this result were only used to evaluate the mobility of many and not single individuals.

#### Travel Time Estimation

The following research is based on passive tracking. Passive tracking uses mobile subscriber events captured in the core network to locate the subscriber. Therefore, passive tracking does not require any equipment in the mobile station or involvement of the mobile subscriber. On the other hand, active tracking requires the participation of the mobile subscriber and the mobile station. Active tracking technologies are Global Positioning System (GPS) and Time Difference of Arrival (TDOA). Systems like TDOA need BTSs that support that feature and therefore require special equipment. These systems have a higher location accuracy than passive tracking system, but they increase either the power consumption of the mobile station (GPS) or the network load (TDOA) or both.

Martin Alger , from Vodafone Germany, was one of the first who investigated travel time estimation on the German higher road network. Double handover (DHO) events were used to compute the cell drive through time, which is the time between entering the cell coverage and leaving it. By estimating the route a subscriber drove within the cell coverage and dividing it by the time difference, the velocity could be computed. An average filter was used that combined the travel times of other subscribers in the same cell to remove stale information. The system was used to investigate the traffic condition on the higher road network in real time. Their research showed that travel times can be estimated for the higher road network without the need of expensive inductive loop detectors. However, their approach focus only on the highway network and does not support the minor road network. The German road network consists of 12.879km highway and 230.517km other roads and therefore their approach is only applicable for 5.59% of the total road network.

To measure the traffic velocity and travel time in Israel, Bar-Gera  used a proprietary system by Estimotion Ltd. The system works with handover events to derive the traveled route and velocity. A sequence of locations derived from the handover footprint is matched to road segments. The length of the road segments together with the estimated handover position was used to derive velocity information. The estimated segment velocity of individual drivers was later combined with velocities from other drivers on the same segment to increase the accuracy and remove noise.

#### Contribution

Our goal is to estimate movement trajectories for cellular network subscribers with a combination of closed and public available datasets. The developed framework allows transforming subscriber information form a network operator into trajectories of subscribers. Our approach works passively and does not need any participation of the subscriber. Therefore, we can estimate trajectories for each subscriber in the network.

We are evaluating how the coverage area for cell sites of a real network can be estimate with limited information. This is different to most research projects  since they are relying on Voronoi diagrams. Part of this research was to evaluate the accuracy of Voronoi diagrams and coverage predictions done with a network planning tool.

Contrary to Bar-Gera  and Alger  we are estimating trajectories for individual subscribers and are not combining them. Additionally our approach works also for the minor road network unlike the system developed by Vodafone.

In contrast to others our trajectories are time-dependent and can therefore react to seasonal changes or different weekdays. This is important since the behavior of the subscriber will be different for example during workdays and weekends or between summer and winter.

Another benefit is that previous incidents in the network can be replayed and simulated by generating trajectories for the particular day with our framework.

# Methodology

In this section, we introduce our approach about how to estimate movement trajectories for cellular network subscribers. At first we are introducing the dataset we are using and additionally data sources we require for the operation of our framework. Followed by a delineation of our approach and the algorithms used to transform the input data into estimated movement trajectories. We describe how the start and end point of a trajectory and its route can be predicted and how to derive the subscribers velocity from handover events. At last we present how the validation of the approaches was done. We introduce the metrics used to evaluate the different aspects of the trajectory estimation such as handover position, route geometry and velocity.

## Dataset

The trajectory estimation process utilizes different types of data in order to derive the subscribers mobility. At first we are introducing the dataset provided by the network operator. This dataset consists of subscriber information (call establishment / termination, handover) and the location of each cell site. Since GSM does not directly expose the subscribers location, rather the current cell site is know we need additional data sources to enhance the positioning. These data sources are used to estimate the border of each cell site, help to predict the start and end position of the trajectory and to derive the subscribers velocity.

### Provided Information

In order to estimate the subscribes mobility we need a record of its behavior. The subscribers behavior can be derived by capturing events in the networks operator core network. The process of capturing subscriber events by installing network probes into the core network is described by Valerio . From the subscriber information we can derive a coarse location of the subscriber by looking at the Cell-Id and Location Area Code (LAC). Together with the timestamp of the event, the mobility can be derived.

An event captured in the core network consists of the following attributes:

- Time stamp, time when the event was captured

- Id, identifies the subscriber

- Cell-id, id of the currently connected cell id

- LAC, location area of the current cell

- Event, defines the captured event

An example for events captured in the core network is depicted in Table [table:events]. Here the first event describes a call establishment, followed by three handover event and at last a call termination event.

The second data, that was provided by the network operator was the location of its cell sites. For each cell site the location specified by the latitude and longitude coordinate of the tower and an angle that defines the main beam direction of the transmitter is defined in the dataset. However, many base transceiver stations (BTS) will share the same location as only the location of the tower for each BTS is known.

### Additionally Data Sources

For now we know the subscribers behavior over time by investigating the events in the event stream and the location of each cell site. However, this data is not sufficient to estimate a movement trajectory for the driver. Therefore the framework needs additionally data sources that will be introduced in this section.

#### Cell Area Estimation

As of now we only know a coarse location of the subscriber defined by the location of the currently connected cell site. An important aspect for the estimation of the movement trajectory is the subscribers’ location. Since the only information that is available is the currently connected cell site, an approximation of the cell sites coverage area is needed. The coverage area for the currently connected cell site narrows down the subscribers location, as it is in general implausible that a subscriber is located outside the coverage area of the cell site. We are investigating two different types of cell area estimation. The first, Voronoi diagrams, is using the location of all cell sites and defining a polygon for each cell site. A subscriber located within this polygon has the closest distance to the cell cite to which the polygon belongs. The second, coverage prediction, is using a network planning tool and is beside taking the location of the cell sites also taking the environment into account. The environment is defined as the elevation of the surface, the transmit power, etc.

A simple approximation of the cell area can be derived from Voronoi diagrams as described by Baert and Seme . Hereby, Voronoi tessellation, is used to partition a plane with n points into n convex polygons . Each of this convex polygons contains one generator point, which in our case are the locations of the cell sites.

However, many of the cell sites consists of more than one BTS and form a sector antenna. Since all of these BTS have the same location, their coverage area would be identical.

We overcome this by moving the location of each sector, based on the angle of the sector, out of the center location of the cell site. The longitude location of the sector will be moved with Equation [eq:sectorX] and likewise will be the latitude location moved with Equation [eq:sectorY]. The constant factor 150000 means a maximum movement of 1.6 meters in either direction.

x=x+cos(α)150000y=y+sin(α)150000

A more accurate method for estimating the coverage area is the use of a network planning tool. Voronoi diagrams, only use the location of each cell site, however, in a real mobile network the coverage area is influenced by a variety of factors such as:

- Network load

- Antenna characteristics

- Transmitter power

- Shadowing

- Landscape characteristics

- Surrounding cell cites

We estimated the coverage area for all cell sites of the Austrian network operator A1 in the research area. The accuracy of the coverage prediction depends on the quality of the input data. Since A1 only provided us with the location and the beam direction angle of its antennas, we were using publicly available datasets, to enrich our coverage estimation. Part of this dataset are digital elevation models, buildings models and a transmitter power database that will be used by the network planning tool for the coverage prediction.

#### Estimation of The Transmitter Power

The coverage area directly depends on the transmitter power of the antenna. We are using transmitter power measurements from the Austrian Forum Mobilkommunikation (FMK) to enhance our coverage prediction. The FMK provides a service where participating mobile network operators can upload information about their infrastructure. Most important for our work was the location and measurements of the transmitter power of cell sites.

However, there are a few limitations while using this data. First, the service provided by the FMK is voluntary and, therefore, the data can either be out of date or absent. Second, only the highest transmitting power of all sectors located at the same cell tower is provided. Last, there is no information available to which network operator the cell site belongs.

In the following paragraph the approach to retrieve transmitter power estimations for each cell site is described. At first we define two sets, one for the cell site locations of the provider and one for the FMK transmitters. Let A=pos0,pos1,...,posi be a set of cell site location of the network operator and B=pos0,pos1,...,posj be a set of cell site locations provided by the FMK. From these two sets we can than derive an intersection C=A∩B of cell site location that are present in both sets. Additionally the set C contains the transmitter power for each cell site from the FMK set B. The intersection performs a filter process and ensures that only cell site locations from the FMK are used which belong to the network operator. To estimate the transmitter power of cell sites not contained in C but contained in A, a scattered interpolation is performed. The scattered interpolation process takes the location and the transmitter power of the set C and interpolates the transmitter power for the set D=A∖B. An example for a scattered interpolation done for 20 random cell site with a random transmit power in the range of 45-52dBm is depicted in Figure [fig:transmitpower].

#### Modeling the Environment

The coverage prediction done with the network planning tool needs a model of the real environment in order to predict the coverage area for each BTS. For now the model consist of the location and the transmit power of each BTS. However, as mentioned before the coverage area of the BTS depends also on the landscape characteristics. A representation of the landscape is defined by a digital elevation model that describes the surface.

The digital elevation model used for the coverage estimation offers a resolution of 25 meters, which means that each pixel covers 25 by 25 meters. During our research we have seen that the resolution of the underlaying elevation model directly affects the quality and, therefore, the accuracy of the coverage prediction.

To improve the coverage prediction further we integrated a building block model for the two research areas. For the city of Linz, the footprint of every buildings is known from OpenStreetMap, though, their height is unknown. For these buildings we assumed a medium height of 15 meters. On the contrary, the city of Vienna provides a building model with height information. The building model of the city of Vienna consists of the footprint of the building and a category that specify its height. The available height categories are:

- 1.6 - 4.5 m

- 4.6 - 7.5 m

- 7.6 - 9.0 m

- 9.1 - 12.0 m

- 12.1 - 16.0 m

- 16.1 - 21.0 m

- 21.1 - 26.0 m

- > 26 m

In our building model we enriched the building footprint from OpenStreetMap for the city of Vienna with the mean height of the specified category.

In total we computed two coverage predictions for each research area. The first uses the transmitter location, the transmitter power and the digital elevation model. The second, enhances the first one by additionally using the building model on top of the elevation model.

We will later discuss the differences and benefits of the three – Voronoi, Cellular planning, Cellular planning with buildings – cell area estimation methods.

Besides modeling the environment for the coverage area prediction we also need a representation of the environment in which the subscriber is moving. In our case we need additionally information to narrow down the users location. Most important is a model of the road network on which the subscriber moves. Since individuals move on predefined ways our framework focuses on ways, paths and streets. The road network we are using is obtained from OpenStreetMap and freely available. OpenStreetMap covers almost all ways and additionally provides other information such as the maximum allowed speed which is later used to validate and adapt the velocity estimation.

As of now the framework knows the road network on which the subscriber can move and the coverage area of the cell sites. However, the location inside the borders of the cell site is still unknown. The location within the cell site is an important figure for estimating the trajectory, in particular its start and end position. To further enhance the localization of the subscriber within the cell site we are using population density and land use information. These informations allow the framework to restrict the subscribers location by defining areas, in which it is unlikely that a subscriber starts or ends his journey. The European Environment Agency (EEA) provides both population density and CORINE land cover maps. The population density map and the CORINE land cover map both have a spatial resolution of 100 meter. Therefore, each pixel covers 100 by 100 meters. We will later discuss how this information was used to estimate the users start and end position.

## Subscriber Mobility Estimation

In the following section, we will delineate the approach used to estimated the subscribers’ mobility and is based on work which has been presented at the European Modeling Symposium 2013 . Here we will explain how the transformation of the input dataset such as subscriber information, coverage prediction, population and land use information into movement trajectories of the subscriber is done. In particular, handover updates are used to locate the subscriber and derive its velocity. Since that we improved the trajectory generation with new coverage predictions and handover point estimations. Additionally we integrated an approach to derive the subscribers’ velocity from handover updates.

The general idea is that we have samples of the subscribers trajectory. At discrete times we know that the user has entered the coverage area of new cell site due to a handover event. We also know that the user started and ended his journey within the first and the last cell site respectively. Still missing, however, is an estimation of a route that traverses each of the cell sites and a location – handover position – to each of the discrete time points to derive the subscribers’ velocity.

### Estimating the Subscribers Path

Each trajectory needs an estimation of the route traveled by the subscriber. All we know from the subscriber information provided by the network operator is a series of handover events that gives a rough representation of the route traveled by the user. The following parts will describe in more detail how the subscribers’ path and velocity can be derived from this information.

The estimation of the subscribers path is an iterative process that takes the cell area estimation, the road network and the subscriber information as input variables. The process first estimates the start and end position of the subscriber and then computes a route between these two points. At the end of the process the velocity of the subscriber will be computed by taking the position and the timestamp of consecutive handovers into account.

#### The Location of the Subscriber

Until now the location within the cell coverage is still unknown. However, for estimating the subscribers route a start and end point is required. The start and end point of a trajectory is marked by the call establish event and the call termination event respectively. Therefore, our approach is predicting the subscribers start and end position within the extent of the cell where the event originated.

To narrow these boundaries, the system uses geographical input (CORINE land cover) and socio-statistical maps (population density maps). By using CORINE land cover data the system can define areas where it is unlikely that a subscriber will start or end his journey. This step can be parameterized by giving each of the CLC classes a percentage factor. The percentage defines how likely it is that a subscriber starts or ends a call in this class. In this manner the subscribers location can be restricted to a few defined CLC classes such as artificial surfaces, urban fabric and industrial, commercial and transport units.

Our second assumption is that subscribers will more likely start or end their call in a higher populated area within the cell boundaries. Therefore, the system uses population density maps in order to better estimate the start or end position.

An example for our assumption is depicted in Figure [fig:populationgrid], were an area defined by its border with three different population density areas is illustrated. Based on our assumption the subscriber will be more likely located in the area with 240[peoplekm2] than in the ones with 45[peoplekm2] and 30[peoplekm2]. Hence, our framework will use all different population density areas within the border and compute a probability density function. After a population area has been chosen by the random process the subscriber location will be uniformly located within the border of the population area. This process ensures that subscribers will be located with a higher probability in areas with a higher population density.

#### The Traveled Route

Since individuals travel most of their time on predefined paths we restrict our generated trajectories to be on top of the road network of the research area. In this manner the subscriber route will be computed on the OpenStreetMap road network between the before estimated start and end position. By default, 20 routes with different start and end position will be computed. Afterwards, these routes will be validated in terms of their geometry and the time it takes to traverse the route.

The first validation calculates the squared sum of the minimum distance between the route and the centroid of each cell areas. This is the same metric that Tettamanti et al.  used for their system. For each of the previously calculated routes j, the squared sum of all minimum distances di,j between the route and the cell site was calculated.

We introduce a duration ratio routeratio between the call duration dcall and the time it takes to drive the route droute as depicted in Equation [eq:ratio]. The ratio gives information whether the estimated route is either too fast or too slow.

Dj=mini=1m(di,j)2routeratio=dcalldroute

The squared sum Dj in combination with the duration ratio routeratio was used to chose the most likely route the subscriber has traveled in terms of distance to cell sites and travel time.

### Deriving Velocity from Handover Positions

To describe the mobility of the subscriber it is important to know its movement over time. In our case the movement is defined by handover events that are issued over time. Whenever a subscriber is in an active call and moves from one cell to another cell, a handover event is issued. This event exposes a coarse location (cell site) and a timestamp. By using this information, the velocity of the subscriber can be derived. The velocity of a subscriber is an important figure for mobility simulation, since the velocity, together with the estimated route, describes the movement of the subscriber. However, as the system only knows the cell in which the handover event originated – cell to which handover was made – an estimation of the actual handover point is needed.

#### Predicting the Handover Position

The framework uses two consecutive handovers to compute the origin of the handover position. In theory, a handover is carried out whenever a subscriber leaves the coverage area of one cell site and enters the coverage area of another cell site. In practice, however, there are more aspects that affect a handover event, besides the coverage area of a cell site. These aspects can be for instance network load, network interference on the channel, etc.

In our research we focus on the handover prediction based on the coverage of the involved cell sites, since there is no information of the network load or the network interference available in the event stream provided by the network operator.

Throughout our investigation of handover sequences we discovered three different types of handovers. The first is defined as an overlapping handover. An overlapping handover is where the coverage areas of the two involved cell sites have an intersection. Note that an overlapping handover can not occur when using Voronoi diagrams for the cell area estimation since Voronoi diagrams are not overlapping. However, when using a network planning tool for the coverage area estimation there are many cell sites whose coverage areas overlap.

The second handover, is an unconnected handover, where the involved coverage areas do not intersect. This means that a handover is made to a cell site whose coverage area the subscriber has not entered yet.

The third, ping-pong handover is where a handover is made from cell site A to cell site B and later back to A. These handovers are an undesirable effect not only for the network itself but also for the timing estimation. Therefore, we filtered out ping-pong handovers by removing the last handover event, in this case the handover event from B to A.

Algorithm [alg:prediction] depicts how the system estimates the handover position for overlapping and unconnected handover. For an overlapping handover the centroid of the intersection C=A∩B of the involved cell site is computed and then the nearest point on the calculated route is set as the handover position.

For unconnected handover the line between the closest points of the two coverage areas is computed, if this line intersects the before computed route then the intersection position is set as the handover position. If this is not the case then the midpoint of the line is computed and the nearest point on the calculated route is set as the handover position. The last step in each of these branches was performed to associate the handover position with the road network.

latex@errorgobble

[H] [alg:prediction] HP←List() ( Handover loop)i←1 length(handover)−1  curr=handover[i]; next=handover[i+1] /*overlapping handover*/

#### Computing the Average Velocity

To derive the subscribers mobility and in generally its movement the velocity needs to be computed. After the handover positions have been estimated the average velocity of the subscriber between two consecutive handover points can be computed. Each of the handover points contains a timestamp, that indicates when the handover event occurred. To derive the velocity, the distance between the two handover points is divided by the time difference of the two timestamps. The distance denotes to the distance on the road network and not to the linear line.

#### Velocity Validation

Due to the deviation between the observed (real) handover position and the estimated handover position, the subscribers velocity can differ. During the development of the framework we discovered that certain handover can lead to a velocity overrun. Hereby, two handover events were made in a short period of time t<10 seconds where the distance between the two handover events is fairly large compared to the small time difference. By computing the velocity between these two handovers the resulting velocity was few times higher than the maximum allowed speed on that particular road. This was affected by a wrong handover position estimation and lead to the development of an adaption algorithm (see Algorithm  [alg:adaption]) that first validates each velocity and later rearranges the handover position in case the computed velocity was higher than a defined threshold.

The general approach is that in case of a velocity overrun the algorithm repositions the handover positions between the current and either the predecessor, the successor or both handover events.

The algorithm iterates over the whole trajectory after the velocity for each handover segment has been computed. A handover segment is a part of the route from one handover position to a subsequent handover position and the average velocity of the segment. For each handover segment, the algorithm compares the velocity with the maximum allowed speed of that particular road. In the case the velocity is 1.7 times higher than the maximum allowed speed, handover repositioning takes place.

The first step is to calculate the distance the subscriber can drive at the maximum allowed speed for the duration of the handover segment. The duration is defined by the timestamps difference of the involved handover. The difference between the computed allowed distance and the distance between the two handover positions corresponds to the handover position deviation. For the current handover segment the velocity will be set to maximum allowed speed. If the handover segment has no predecessor, but a successor then the difference between the computed allowed distance and the distance between the handovers will be added to the successor. Likewise, if the handover segment has a predecessor but no successor the reposition works the same just the successor is replaced by the predecessor.

If, however, the handover segment has both a predecessor and a successor, which is the case for most handover segments, then the algorithm repositions either the first handover position, all two handover positions or the second handover position. At first the algorithm is equally adding the difference between the allowed distance and the distance of the handover segment to the predecessor and successor. In the case that the velocity of the predecessor is greater than the maximum allowed speed after repositioning, the reposting of the predecessor will be withdrawn and the repositioning will only affect the successor. In equal measure, this will be done to the successor if its velocity exceeds the maximum allowed speed.

latex@errorgobble

[H] [alg:adaption] ( Adaption loop)i←1 length(trajectory)  allowedSpeed<-MaxSpeed(trajectory[i]) c=trajectory[i]p=trajectory[i-1]n=trajectory[i+1] /*speed overrun*/

## Evaluation of Subscriber Trajectories

To evaluate our findings we define a set of measurements that will be computed for each of the generated movement trajectories. To validate the route geometry, the handover position and the velocity of the subscriber we recored a GPS track for each test ride. Additionally we stored the position of each handover by tracking changes in the Cell-Id and LAC on the handset and storing the location when the subscriber is connected to a new cell site. From the GPS track and the timestamp information the velocity of the subscriber can be derived and later be compared with the estimated one. The velocity evaluation compares the average velocity of the subscriber between two handover points with the estimated velocity. GPS was only used during research in order to validate our approaches and assumption.

For our investigation we generated and evaluated trajectories for mobile network subscribers for three – Voronoi diagrams, Network planning tool and Network planning tool with buildings – different coverage estimation methods.

### Route Geometry

An important aspect of the trajectory generation process is to identify the route traveled by the network subscriber. In order to evaluate our approach of computing the route between a computed start and end position we used two different metrics. This metrics are the Hausdorff distance and the Fréchet distance and were used to compute the similarity between the traveled (GPS) and the estimated route.

To compute the maximum deviation between the estimated route and the actual subscriber route the Hausdorff distance was computed. The Hausdorff distance is defined as the greatest of all the distances from a point in one set to the closest point in the other set. In our case X and Y are the estimated and the actual route respectively.

dH(X,Y)=max{ supx∈Xinfy∈Yd(x,y), supy∈Yinfx∈Xd(x,y) }

The Fréchet distance was introduced by Fréchet  and is well-suited for the comparison of trajectories since they take the continuity of the trajectory into account. In our evaluation we are using the algorithm by Alt and Godau  that computes the Fréchet distance on two polygonal curves in Euclidean space.

The Fréchet distance for two curves A,B:[0,1]→V, in our case two routes, is defined as

$$\delta_F(f,g)=\inf_{\substack{\alpha [0,1] \rightarrow [a,a'] \\\beta [0,1] \rightarrow [b,b']} }\,\, \max_{t \in [0,1]} \,\, \lVert f(\alpha(t))-g(\beta(t)) \rVert$$

where α and β are continuous and increasing functions with α(0)=a, α(1)=aʹ, β(0)=b and β(1)=bʹ.

Our evaluation was done using Christophe Genolis implementation of the Fréchet distance.

### Velocity Comparison

A trajectory consists of the traveled route as well as timestamps that define when a point on the route has been traversed. This information describes the velocity of the subscriber on the road. In order to evaluate our approach, to estimate the subscribers velocity based on handover positions, we compare the estimated velocity with the ground truth velocity obtained by the GPS information. Another aspect was to evaluate the performance of the adaption algorithm that repositions handovers in case a velocity overrun happened.

One of the metrics is the mean absolute error (MAE )between the observed average velocity Yi and the computed velocity Yi. The observed average velocity Yi is a subset of the velocity obtained from GPS between the two consecutive handover positions of the researched handover segment. The computed velocity is the velocity derived from computing the distance between the consecutive handover points divided by the timestamp difference. We also used the root mean square error (RMSE), that is stronger influenced by large errors than by small errors.

MAE=1n|Yi−Yi)|i=1n

RMSE=(i=1nYi−Yi)2n

Where n is the amount of handover segments for which the velocity was computed with Algorithm [alg:adaption].

### Handover Deviation

Since we estimate the handover position based on the geometry of the coverage area of the involved cell sites we need a metric that indicates how far the estimated handover positions is away from the observed ground truth handover position. The Euclidean distance between the estimated handover position and the observed handover position is used as a metric. This metric is used to evaluate the handover position prediction as well as the coverage area estimation that is used by the handover position prediction algorithm.

# Experiments and Results

After we delineated our approach, for the estimation of movement trajectories for cellular network subscribers, we now present our results and the environment in which they were performed. The here presented results were gathered during four test drives in urban and semi-rural areas in Austria. The first part will introduce the environment in which the test drives were performed and the equipment that was used. The second part presents the gathered results in terms of route geometry, handover deviation and velocity estimation. We will show that is possible to estimate valid and reliable trajectories for cellular subscribers.

## Environment

In this section we will present the environment in which the four test drives took place and the information that was captured by handset. For each of the test drives we were using the same handset with the same subscriber identity module (SIM). The handset in use was a Samsung S3 running Android 4.1.1 JellyBean. As mentioned before the handset was used to initiate a call with a second subscriber and maintain the call for the duration of the test ride. During the call the handset stored the location of the subscriber by using GPS and additionally the current connected cell site (Cell-ID and Location Area Code). The network operator later provided subscriber information for each test drive that was consumed by the framework to estimate the trajectories.

Table [table:tracks] gives an overview of the start and end position and the amount of handover events for each of the four test rides. The amount of handovers in brackets correspond to unfiltered raw events. The one outside are filtered handover where the framework removed ping-pong handover. We The first test drive started in the city of Linz and ended in the village Treffling that is located in the outskirts of Linz. This test drive was performed entirely on the higher road network and features a semi-rural scenario that consists of an urban and rural part. The second test drive took place within the borders of the city of Linz. The test drive was performed on streets of the minor road network as well as on the city highway. The third and fourth test drive took place within the borders of the city of Vienna. Both test drives were performed on the major and minor road network.

In Figure [fig:563] and [fig:144] the first and the fourth trajectories are illustrated. Each of the three coverage prediction methods – Network planning tool (a), Network planning tool with buildings (b) and Voronoi diagrams (c) – along with the estimated handover position is depicted. The last Figure (d) shows the route of the test drive with the exact location of each handover. Due to the limited place the second and third trajectory was omitted.

## Results

After introducing the environment in which we performed our test drives, we will now present the results gathered during the test drives. We divided the results into three categories –route geometry, handover deviation and velocity estimation – that will be delineated in the following sections. The results indicate that the subscribers route, the handover position and the velocity can be estimated for the purpose of mobility simulations.

### Route Geometry

To validate the similarity of the estimated route and the actual route the before mentioned discrete Hausdorff distance and the Fréchet distance were used. While the Fréchet distance is better suited for curve matching the Hausdorff distance gives us the maximum distance between the estimated and the actual route. Hereby, the Hausdorff either indicates the maximum distance between the estimated start and end positions or the maximum distance between the estimated and the actual route. The results in Table [table:routesim] show a minimum and maximum Hausdorff distance of 0.0075∘ (590m) and 0.0151∘ (1189m) respectively. The minimum and maximum Fréchet distance of 0.0012∘ (94m) and 0.0047∘ (370m) respectively. Hence, all the estimated trajectories show a high similarity with the actual route and are therefore suited to describe the subscribers mobility.

### Handover Deviation

The accuracy of velocity estimation directly depends on the handover estimation. Therefore, it is important to validate the handover position algorithm. We compared the handover positions estimated with the three coverage prediction method against the real handover position stored by the handset. Though Voronoi diagrams only takes the location of the cell sites into account, the handover position could be estimated reasonably well. We saw that Voronoi diagrams worked best in urban areas where the density of cell sites is high. In rural areas, with less cell sites, the Voronoi diagrams became fairly large since the extent is only limited by the surrounding cell sites.

In Figure [fig:handoverdeviation] the handover deviation for the first trajectory is illustrated for each handover position. It can be seen that the deviation error increased in the second half of the plot. This was caused by leaving the urban area and driving into a suburban one. Here, Voronoi diagrams performed worse compared to the other coverage predictions.

Table [table:handover] gives a statistical overview of the handover deviation between the estimated position and the observed one. All handover positions have been estimated with Algorithm [alg:prediction] and were performed with the three coverage prediction methods. Figure [fig:handoverdeviation] illustrates the handover deviation for each handover at each trajectory and for each coverage prediction.

By combining the handover deviations over all four trajectories and computing the MAE and RMSE, the coverage predictions without buildings performed best with a MAE of 0.176km and RMSE 0.231km. Followed by the coverage prediction with buildings MAE 0.189km and RMSE 0.253km and the Voronoi diagrams with MAE 0.238km and RMSE 0.311km.

For the fourth trajectory the minimum, maximum and mean handover deviation was 0.010km, 0.802km and 0.328km respectively which shows that the handover position can be estimated with high accuracy.

The third trajectory shows a high maximum handover deviation that was caused by network that was not performing a handover while driving for 3.55km within the city of Vienna. Since the handover algorithm computed a handover position between the two cell site coverage areas, that were far-off, a large handover deviation occurred.

### Velocity Estimation

In this section we want to discuss the results of the raw velocity estimation and the adapted velocity estimation with handover repositioning. We compared each of the velocities against the velocity derived from the GPS information. Since the estimated velocity accords to the average velocity between two handover position we computed the average GPS velocity between this two handover points. In Figure [fig:velocity] the raw estimated velocity for the first trajectory is illustrated. It can be seen that there occur many velocity overruns for eacht of the 3 coverage estimation methods. By comparing this Figure with the handover deviation (see Figure [fig:handoverdeviation]) it is clear that a velocity overrun happens when a large (≥0.5km) happens. Contrary Figure [fig:velocityadaption] shows the estimated velocity with adaption and handover repositioning. Hence, we see that the peaks could be reduced. To validate the accuracy, the MAE and RMSE for both velocity estimations for each trajectory and each coverage prediction methods was computed. The results depict in Table [table:results] show that the adaption algorithm can reduce the error introduce by a wrong handover position estimation.

For the coverage prediction without buildings the velocity MAE, by using the adaption algorithm, was reduced on each trajectory by an average of 60.40%. For the coverage prediction with buildings the reduction was 65.75% and for Voronoi diagrams 73.01%.

However, this indicates only how well the adaption algorithm work but not how accurate the velocity estimation is in general. To validate the velocity accuracy for the three different coverage prediction methods the absolute adapted velocity errors from the four trajectories were combined to compute the MAE. Here the coverage prediction with buildings had the minimum absolute error for all trajectories with 14.887km/h. Followed by the Voronoi diagrams with 14.921km/h and the coverage prediction without buildings 16.004km/h.

This indicates that post-processing in the form of the adaption algorithm is necessary to estimate an accurate velocity.

# Conclusion and Discussion

We have shown that trajectories can be estimated for cellular subscribers by using subscriber information captured in the core network and the location of cell sites. We presented two methods for the cell site coverage estimation and compared them in terms of handover position and velocity estimation. Our results showed that the handover position can be estimated with a mean deviation of 179m by using the coverage prediction without buildings. To minimize the effect of a deviated handover position we developed an adaption algorithm that can reduce velocity overruns and decrease the mean absolute error. The validation has shown that the adaption algorithm reduces velocity overruns at all four test drive trajectories. The mean absolute velocity error over all four trajectory was 14.887km/h when using coverage prediction with building models. This indicates that the velocity can be precisely estimated for urban and rural areas.

The gathered results have shown that cellular network subscriber trajectories can be generated for the purpose of mobility simulations. They are well-suited to describe the subscribers mobility since they cover the traveled route and the velocity.

A future enhancement would be to create a better model of the mobile network to enhance the coverage prediction. This can be achieved by integrating accurate transmitter power and antenna characteristics that was provided by the network operator.

# Competing interests

The authors declare that they have no competing interests.

# Author’s contributions

Text for this section …

# Acknowledgements

Text for this section …

# Figures

[fig:transmitpower] image

[fig:populationgrid] image

[fig:handover] image

[h!] [fig:563]

[h!] [fig:144]

[fig:handoverdeviation] image

[fig:velocity] image

[fig:velocityadaption] image

# Tables

[h]

| Time stamp | Id | Cell-Id | LAC | Event |
| --- | --- | --- | --- | --- |
| 1327678833 | 4 | 51520 | 5501 | Call Establishment |
| 1327678854 | 4 | 31510 | 5501 | Handover Update |
| 1327678935 | 4 | 31210 | 5502 | Handover Update |
| 1327678949 | 4 | 40510 | 5502 | Handover Update |
| 1327679027 | 4 | 40510 | 5502 | Call Termination |

[table:events]

[h]

| # | Start [Lat,Lon] | End [Lat,Lon] | Handover |
| --- | --- | --- | --- |
| 1 | 48.28105,14.30415 | 48.33499,14.3780 | 18 (22) |
| 2 | 48.31616,14.29052 | 48.28169,14.30275 | 17 (19) |
| 3 | 48.13993,16.32367 | 48.19880,16.25597 | 16 (20) |
| 4 | 48.19283,16.27260 | 48.15278,16.30172 | 27 (37) |

[table:tracks]

[h]

| Trajectory | dH(X,Y) | δF(f,g) |
| --- | --- | --- |
| 1 | 0.0135∘ | 0.0047∘ |
| 2 | 0.0075∘ | 0.0015∘ |
| 3 | 0.0151∘ | 0.0021∘ |
| 4 | 0.0107∘ | 0.0012∘ |

[table:routesim]

[h]

| # | M | min | Q1 | x | x | Q3 | max |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | P | 0.161 | 0.468 | 0.663 | 0.705 | 0.878 | 1.403 |
| 1 | PB | 0.101 | 0.547 | 0.716 | 0.711 | 0.928 | 1.315 |
| 1 | V | 0.142 | 0.420 | 0.837 | 0.715 | 1.023 | 1.155 |
| 2 | P | 0.219 | 0.286 | 0.321 | 0.401 | 0.483 | 0.900 |
| 2 | PB | 0.132 | 0.261 | 0.346 | 0.399 | 0.525 | 0.884 |
| 2 | V | 0.048 | 0.229 | 0.343 | 0.386 | 0.482 | 0.900 |
| 3 | P | 0.111 | 0.360 | 0.449 | 0.677 | 0.604 | 3.786 |
| 3 | PB | 0.148 | 0.338 | 0.451 | 0.679 | 0.536 | 4.062 |
| 3 | V | 0.013 | 0.281 | 0.409 | 0.669 | 0.639 | 4.085 |
| 4 | P | 0.020 | 0.223 | 0.309 | 0.355 | 0.504 | 0.863 |
| 4 | PB | 0.030 | 0.213 | 0.325 | 0.359 | 0.506 | 0.850 |
| 4 | V | 0.010 | 0.175 | 0.321 | 0.328 | 0.443 | 0.802 |

[table:handover]

[h]

| # | M | MAE | RMSE | adaptMAE | adaptRMSE |
| --- | --- | --- | --- | --- | --- |
| 1 | P | 42.365 | 62.593 | 23.360 | 31.514 |
| 1 | PB | 50.315 | 69.985 | 23.127 | 29.142 |
| 1 | V | 59.704 | 85.549 | 17.648 | 25.250 |
| 2 | P | 81.304 | 138.707 | 8.970 | 16.047 |
| 2 | PB | 60.964 | 99.090 | 9.436 | 12.086 |
| 2 | V | 113.999 | 181.854 | 13.029 | 16.932 |
| 3 | P | 21.125 | 26.808 | 13.305 | 17.799 |
| 3 | PB | 26.384 | 33.786 | 11.941 | 16.607 |
| 3 | V | 28.449 | 40.088 | 13.036 | 18.781 |
| 4 | P | 58.425 | 150.199 | 17.081 | 22.000 |
| 4 | PB | 48.061 | 125.747 | 14.553 | 17.795 |
| 4 | V | 72.715 | 191.037 | 15.388 | 21.599 |

[table:results]

Statistic about the German road network January 1, 2013 http://www.statistik-portal.de/statistik-portal/de_jb16_jahrtab36.asp

The Digital Elevation Model over Europe from the GMES RDA project http://www.eea.europa.eu/data-and-maps/data/eu-dem

More information about how building information can be retrieved from OpenStreetMap http://wiki.openstreetmap.org/w/index.php?title=Buildings&oldid=1050850

The GIS service from the city of Vienna is located here https://www.wien.gv.at/kultur/kulturgut/architektur/gebaeudedaten.html

Population density disaggregated with Corine land cover 2000 http://www.eea.europa.eu/data-and-maps/data/population-density-disaggregated-with-corine-land-cover-2000-2

Corine Land Cover 2006 seamless vector data http://www.eea.europa.eu/data-and-maps/data/clc-2006-vector-data-version-3

More information about the implementation of the Fréchet distance http://www.inside-r.org/packages/cran/longitudinalData/docs/distFrechet